Electric Circuits Laboratory

- WIRE, Bb, DMM (measure R & V) (1.5h)

In this lab you will learn how to work with wire, resistors, and the solderless breadboard.  You will use the digital multi-meter (DMM) to take resistance and voltage measurements.  Refer to the CT GUIDE on the following topics: wire, breadboard, resistors, potentiometers, the trainer, DMM use, and continuity testing.

Note, when using the DMM, always place the red test lead in the "Hz V" port.  For this class we will never measure electric current so never put any test lead in the "mA" or "10A" ports.

  - EQUIPMENT LIST

      - Trainer system (XK-700) (for the breadboard (Bb) and power supply, possibly the pot)

      - Power supply (and power jack adapter) (10v)

      - Solderless breadboard

      - Wire cutter/stripper

      - Digital multi-meter (DMM)

      - Resistors								Nominal   Measured (k)

1.8 k:  (brown – gray – red) 			_______   _______

2.0 k:  (red – black – red)				_______   _______

100 k: (brown – black – yellow)			_______   _______

      - Potentiometer (1 k or similar, for continuity threshold test) (small screwdriver)

      - Spool wire (a spool of wire will be kept in front of the lab.  obtain a couple of pieces)

      - Jumper wire kit (22 AWG wire)

      - Wire gauge tool (just a few are available so students must share, bring to front desk after using it)

  - VERIFY RESISTOR VALUES

First determine the resistor nominal values from the color bands.  Refer to the CT GUIDE on the 4-color band resistor code (section 6.3).  But then ALWAYS measure R using DMM to verify it is correct.

(or any other component too like capacitors or inductors)

(***) Record for each R:  (1) color bands, (2) corresponding number, & (3) "nominal" resistance (in k).

record data on ANSWER SHEET!!

Note for resistors: if the resistor leads are bent up badly, then discard it and get a replacement.

  - WIRE SIZE

Here we will cut and strip some wire, then measure wire size and verify its AWG value.  BE CAREFUL when using the wire cutter/stripper.  Always keep your fingers away from the cutting area.  Only use 22 AWG wire or smaller (smaller wire has BIGGER numerical AWG value, like 24 AWG) with our breadboards.

STEPS

      - Use a wire cutter/stripper tool to cut a 2 to 3 inch piece of wire from the spool.

      - Strip (remove) about 1/2" of insulation.  Do not cut the metal underneath.

      - Measure the wire size using the caliper (not a standard tool in a circuits lab).  Calipers must be shared.

Return the caliper to the designated area when done.

      - (***) Convert measurement to AWG value (see table below or look in CT guide or online).

AWG#	Wire Diameter (in)		AWG#		Wire Diameter (in)

18		.0403				21		.0285

19		.0359				22		.0253

20		.0320				23		.0226

24		.0201

  - BREADBOARD

Here we will verify the solderless breadboard's connectivity using CONTINUITY testing feature of the DMM.  First REFER to CT GUIDE on breadboards (section 6.2).

STEPS

Verify the connectivity of the following.  Note whether the DMM produced a "tone" or produced "no tone".

(***) Record results.

  - INSERTING WIRE INTO BREADBOARD

Refer to the CT GUIDE regarding the insertion of wire into a solderless breadboard.  Now practice inserting a resistor and a piece of spool wire into the breadboard as shown.

  - MEASURE R WITH DMM

Here you will measure resistance using the DMM.  Here we will investigate different ways of holding the resistor to get the best accuracy.

    - HOLDING A RESISTOR

Measure the 100k resistor while holding the resistor these different ways

      - DRY fingers - holding the resistor leads against the DMM test leads using your fingers.  Make sure your fingers are touching the DMM test leads while doing this.

      - WET fingers - same as above but make sure you fingers are wet (there's a faucet/sink in the lab).

      - BREADBOARD - insert R into the breadboard (remember the leads must be in different terminal strips).  Then touch the leads with the DMM test leads.

RECORD DATA

______________ DRY FINGERS

______________ WET FINGERS

______________ BREADBOARD

(***) Which method was most accurate?

Note the DMM "computes" R by applying a small known current across its terminals & measuring the amount of voltage that forms across the resistance.

(***) Knowing this, explain how the "bad" method is producing the erroneous measurement?

    - DMM RANGE SETTINGS

Measure the 1.8k resistor using different range settings on the DMM.  Record the displayed value below.  This exercise applies to manual ranging DMM's.  There are also auto-ranging DMM's which select the range automatically.

Displayed value		DMM range setting

_____________		__________

_____________		__________

_____________		__________

_____________		__________

_____________		__________

Which range setting was best for measuring R and why?

Why not just pick the highest range setting to ensure no overload occurs?

(***) Record results.

    - MEASURE RESISTORS WITH DMM

Now measure each of the resistors using proper holding technique and proper DMM range.  Include a calculation of percent difference ("% diff" = (measured - nominal)/nominal).

(***) Record results.

In all future labs always double check the color bands to verify you have the correct resistors.  Also measure the resistance of each resistor using the DMM.  Do this at the START of the lab and write the record the results.

  - CONTINUITY THRESHOLD

The DMM can also check "continuity" between 2 points in a circuit.  Continuity means that resistance between 2 points is below some small threshold (e.g., if a wire connects the 2 points).  If there is resistance is higher than the threshold then there is NO continuity (e.g., if the wire is broken).  The DMM tests continuity just like with resistance measurement, except that it sounds a tone if the measured R is below some threshold.  It sounds no tone if R is above the threshold.  Here we will find out roughly what this threshold is for our DMM using a potentiometer (or "pot").  The figure shown shows a pot mounted on the breadboard, but you can also use the pots on the trainer.  It is important that one DMM lead go to the pot wiper.

STEPS

      - Set the DMM to continuity testing ( or similar).

      - Touch the DMM test leads together.  The DMM should tone.  This is commonly done as a first step.

      - Test continuity across resistor R3. (tone or no tone?)

      - Center the wiper of a potentiometer (1 k or similar) by turning it with a small screwdriver. (or use the potentiometer on the trainer)

      - Test continuity across the pot wiper and one of the other pot leads (either one).  It should not tone.

      - Slowly turn the wiper to one side.  Usually the DMM in continuity mode will show resistance value.

      - Slowly turn the wiper to one side so the resistance drops.  Do this until the DMM tones.

      - (***) Record the threshold value.

  - MEASURE DC VOLTAGE WITH DMM

Now you will measure DC voltage using the DMM.  We must select the proper variable type (DC voltage... don't pick AC voltage), and then the voltage range on the DMM.

STEPS

      - Turn power OFF on trainer.

      - Wire up the circuit as shown.

      - You will draw DC voltage from the regulated power supply on the trainer.

      - Notice the "hot" wire goes to the SECOND terminal on the trainer's power supply, not the first terminal.  The second terminal is controlled by the + voltage dial.

      - Adjust the (+) voltage dial so voltage is 10V.

      - Set the DMM to measure DC voltage with the proper range setting.

      - Touch the red tip to (+) wire and black tip to () wire.

      - Suggestion – a good technique for measuring with the DMM is to strip a little extra insulation from the wire (say, 5/8").  After inserting the wire into the Bb, some bare metal will show.  Touch the DMM test lead to this exposed wire.

      - (***) Record result.

      - Swap the positions of the DMM test leads: touch red tip to () wire and black tip to (+) wire.

      - (***) What is the DMM reading now?

      - (***) What does this tell you about how the DMM measures DCV?

  - SHORTING POWER SUPPLIES

NEVER connect different "source" voltages to each other directly.  This includes any equipment that produces a voltage signal (e.g., DC power supply on trainer, any power supply, function generator output, etc.).  This effectively shorts the power supply.  Short circuits are dangerous and can damage equipment or components (or at least blow a fuse).  Some power supplies have overload protection, but many do not.

(***) Initial on answer sheet indicating you understand.  If not, please talk to me.

  - FINISHING & FOLLOW UP

Clean up your area and check out with the instructor (sign out is on the answer sheet).

Transfer a portion of your answers (as indicated) to the answer sheet (only the answer sheet is turned in).

Complete the follow-up questions on the answer sheet.

LAB 2 – ANSWER SHEET

WIRE, RESISTORS, DMM, BREADBOARD, & MEASURING RESISTANCE

Team Number: ___________

Name 1: _____________________________	Name 2: _____________________________

2.2 Resistor Color Codes

Color bands					Numbers			Nominal value (k)

R1 =                            .	     . x 10^    .     .	          .

R2 =                        .		     . x 10^    .     .	          .

R3 =                            .	     . x 10^    .     .	          .

2.3 Wire Size:        AWG

2.4 Breadboard

Verify the connectivity of the following.  Note whether the DMM produced a "tone" or produced "no tone".

1.            .  A single terminal strip.

2.            .  Between different terminal strips

3.            .  Between terminal strips aligned across the notch.

4.            .  Within a single bus strip

5.            .  Across 2 adjacent bus strips

2.6.1                .  Best way to hold R while measuring resistance?

Why do the other methods produce error?

2.6.2 Best range setting

2.6.3 Measure Rs with DMM

Nom value (k)	Measured R	% diff			DMM range setting used

_      __		_     _	_     __		_   ____

_    ____		_     _	_     __		_   ____

_      __		_     _	_     __		_     __

2.7 Continuity Threshold

.  Record the threshold value

2.8 Measure DCV with DMM

.  (step 9) Record DCV

.  (step 11) Record DCV with test leads reversed

.  What does this tell you about how the DMM measures DCV?

2.9 No short circuits... I understand (initial here):